[image: image2.jpg]N B




SEMP-7.1.2-I03-F01                 领港卡检查清单
                           PILOT CARD CHECKLIST

	Port Name  WELLINGTON     
	Date   29-APR-2026           

	Call sign  V7RS6              
	Dead weigh  38852.4        tonnes
	Year built      2017        

	Draught  aft  4.42    m/     ft     in,
	Forward  6.63   m/  ft      in,
	Displacement  23887   tonnes

	
	SHIP’S PARTICULARS
Length overall  182  m,   Anchor chain: Port  11   shackles,      Starboard   12   shackles,
Breadth       30   m               Stern   NA   shackles
Bulbous bow   Yes/No                               (1 shackle =  27.5  m/  15  fathoms)
	

	[image: image1.png]:E: m =E=
— m L Air
‘ r~i draught
.l _____________ Manifold ft
l4— Parallel W/L —»
Loaded m

Ballast

m







	
	Type of engine WARTSIL 5RT-FLEX50-D TIER II                    
	Max. power    7044       kW (   9446    HP) 
	

	
	Manoeuvring engine order
	Rpm/pitch
	Speed (knots)
	

	
	
	
	Loaded
	Ballast
	

	
	Full ahead
	72/0.7
	10.0
	10.5
	

	
	Half ahead
	50/0.7
	6.0
	6.5
	

	
	Slow ahead
	44/0.7
	4.0
	4.2
	

	
	Dead slow ahead
	35/0.7
	3.0
	3.0
	

	
	Dead slow astern
	-35/0.7
	 Time limit astern
	        30  min
	

	
	Slow astern
	-44/0.7
	 Full ahead to full astern
	      490   s
	

	
	Half astern
	-50/0.7
	 Max. no. of consec. starts
	            12    
	

	
	Full astern
	-72/0.7
	 Minimum RPM  35   
	    3.0  knots
	

	
	
	
	 Astern power
	    70 % ahead
	

	                          BARRED SPD RANGE  53—65 RPM
STEERING PARTICULARS
Type of rudder     SEMI-SPADE/NACA       Maximum angle        35°                 

Hard-over to hard-over     1SET:22.88    2SET: 11.82 s
Rudder angle for neutral effect    ±1.5°                 。
Thruster:   Bow    NA     kW (           HP)     Stern    NA    kW (          HP)



	

	CHECKED IF ABOARD AND READY

	
	Anchors
	(
	Indicators:
	
	

	
	Whistle
	(
	Rudder
	(
	

	
	Radar    ( 3cm
	( 10cm
	Rpm/pitch
	(
	

	
	ARPA
	(
	Rate of turn
	(
	

	
	Speed log  (
	Doppler: Yes/No
	Compass system
	(
	

	
	Water speed
	(
	Constant gyro error
	+   0.3      。
	

	
	Ground speed
	(
	VHF
	(
	

	
	Dual-axis
	(
	Elec. pos. fix. system
	(
	

	
	Engine telegraphs
	(
	Type
	  GP-150           
	

	
	Steering gear
	(
	
	
	

	
	Number of power
units operating
	3
	
	
	

	 OTHER INFORMATION:

Yes
No

1

Is the ship equipped with an engine or shaft power limiter?  EPL or ShaPoli
√
If yes, is the limiter mechanical or software based?  Limiter mechanical / Software based  
√
• If the ship is equipped with a mechanical limiter, has it been removed or disabled prior to the pilot boarding the ship?
N/A
• If the ship is equipped with a software-based limiter, will the ship’s crew be able to override it immediately at the request of the pilot?
√
2
Can the main engine/s attain the posted manoeuvring power (RPMs) without delay?
√
3
Are the master and all bridge navigational watchkeepers familiar with the override feature for the EPL?
√
4
Does the vessel have a risk assessment with appropriate control measures in place for immediate access to the EPL when full manoeuvrability is needed for safe passage?
√
         LI ZENGLIANG                                                                
    Signature of Master                                  Signature of Boarding Pilot



36.37





44.94M





155.5M





26.5M





30.0M








PAGE  
2/2

