| Complete with most up-to-date information and hand over to pilot by Master and make entry in bridge movement book. |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DEP ./ Port | MARSDEN POINT ,NZ |  |  |  |  | Date |  | 1-Feb-2025 |  |  |  |
| SHIP’S PARTICULARS |  |  |  |  |  |  |  |  |  |  |  |
| Name | Port Alberni |  |  |  | Call sign |  | VRDT3 |  | IMO No. |  | 9335886 |
| Deadweight | 17552 | Year built |  | 2006 | Length OA |  | 175.53 |  | Breadth |  | 29.4 |
| Displacement | 24993 | Bulbous Bow |  |  | Yes/No |  | GRT/NRT |  |  | 19831/11677 |  |
| Draught fwd | 9.70 m | Draught aft |  | 10.09m | Draught amidships |  |  |  | 9.88 m |  |  |
| Freeboard | 3.82 m |  |  |  |  |  |  |  |  |  |  |
| Propeller Immersion Draught |  |  | 5.75 m |  | Cargo /Quantity |  |  |  | 31813 mt |  |  |
| Port anchor | 11 Shackles |  |  |  | Stbd anchor |  |  |  | 11 Shackles |  |  |
| 1 shackles=27.4 m/15 fathoms One fathom = 6 feet |  |  |  |  |  |  |  |  |  |  |  |

Air draft

31.91m   ( aft )

ft       Inch	42m

(fwd)

| ENGINE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type of Engine | MITISUBISHI - 6UEC52LA | EPL Implemented * | YES / NO |  |
| Max. Continuous Power (CSR) | 6,840 KW | Maximum Power after EPL | 4,676 KW |  |
|  | RPM | Loaded Speed | Ballast Speed |  |
| Full ahead | 97 | 10.7 | 11.1 |  |
| Half Ahead | 63 | 7.0 | 7.4 |  |
| Slow ahead | 52 | 5.7 | 6.0 |  |
| Dead Slow ahead | 42 | 4.6 | 4.8 |  |
| Astern power |  | ____70_______ % of Ahead power |  |  |
| Dead Slow Astern | 42 | *EPL can be overridden in 1-2 mins, when requested by Pilot. |  |  |
| Slow Astern | 52 |  |  |  |
| Half Astern | 63 |  |  |  |
| Full Astern | 97 |  |  |  |
| Engine Critical RPM | 72-88 | Maximum Number of Consecutive engine Starts |  | 12 |
| Time full ahead to full astern | 7.7 minutes | Time limit astern | 30 minutes |  |
| Rudder Type | Balanced | Maximum Angle | 35 |  |
| Time from hard-over to hard-over: | 25Seconds | Minimum Steering Speed:4kts |  |  |

Equipment Checked and Ready for Use

| Anchors: | Cleared away: YES/NO |
| --- | --- |
| Compasses: | Yes |
| Compass error: | 0 |
| Speed log: | Doppler: YES/NO, Speed: Water/Ground |
| Echo Sounder | Yes |
| GPS: | Type:JRC/JLR7800 |
| ECDIS: (Assigned for pilot’s use ) | Make: Transas Location/No.:NO.1or 2 |
|  | ENC available and updated. ECDIS Alarm & Safety frame On. Safety Depth_11.02 m, Safety Contour _11.02 _ m ECDIS Display Mode: Custom / “All” Display |
| X-Band radar: | ARPA: YES/NO |
| S-Band radar: | ARPA: YES/NO |
| VHF (including handheld): | Yes |
| Steering gear: | Number of power units in use: Both |
| Engine telegraphs: | Yes |
| Rudder / RPM / ROT indicators: | Yes |
| Mooring winches and line: | Yes |
| Navigation lights | Yes |
| Whistle | Yes |

| Equipment operational defects, ship handling and maneuvering limitations, if any: Nil |  |  |  |  |
| --- | --- | --- | --- | --- |
| OTHER IMPORTANT DETAILS (e.g. ship windage area, position of automatic Identification System (AIS) antenna, safe working load (SWL) of bollards), tug push markings on hull Max SWL 64T BOLLARD F&A: 64MT , MAIN DECK 52MT |  |  |  |  |
| Maneuvering Characteristics in Shallow Waters - Advance, transfer and stopping distance of the vessel will considerably increase in shallow waters to > 2 times of the value in deep waters, other external factors remaining constant,) |  |  |  |  |
| Advance _1900ft__ | Transfer_900ft____ |  | Stopping Distance (F. Ahead to F. Astern) 3750ft |  |
| Propeller |  | Right / Left handed | Gyro Error : º High (+) / Low (-) | 0 º H / L |
| Manoeuvring on ships fitted with bridge control: - Operation may be done using Bridge control after risk assessment by Master and Chief Engineer except for JNS vessels. - C/Engineer shall ensure that the ME is tested on Bridge and ECR control both ahead and astern prior manoeuvring and then changed to Bridge or ECR control as appropriate. |  |  |  |  |

| Duty Officer: Name / Sign | Master: Name / Sign | Pilot : Name / Sign |
| --- | --- | --- |