| SHIP PARTICULARS |  |  |
| --- | --- | --- |
| Name: M.V WECO JOSEFINE |  | Call Sign: 9V5717 |
| Displacement: 33661 MT | DWT: 38592 MT | Year Built: 2018 |
| Length: 179.99 m | Beam: 30.00 m | Bulbous Bow: No |
| Draught Fwd: 10.20 m | Draught Aft: 10.89m | Draught Amidships: 10.64 m |
| Air Draught: 34.73m | Port Anchor: 12 Shackles | Stbd Anchor: 11 Shackles |

1 shackle = 27.4m/15 fathoms

30.0

23.9

43.37

156.09

| MAIN ENGINE |  |  |  |
| --- | --- | --- | --- |
| Type: MITSUI MAN-B&W 6S50ME-C8.5X1SET | Max Power: 6780 KW | Max Power: 9218 HP |  |
| Full Ahead: | 80 RPM/Pitch | Loaded Speed 11.3 (kts) Sp | Ballast Speed 11.8 (kts) |
| Half Ahead: | 70 RPM/Pitch | 9.9 | 10.3 |
| Slow Ahead: | 52 RPM/Pitch | 7.3 | 7.7 |
| Dead Slow Ahead: | 42 RPM/Pitch | 5.3 | 6.2 |
| Dead Slow Astern: | 42 RPM/Pitch | 5.9 | 6.2 |
| Slow Astern: | 52 RPM/Pitch | 7. | 7.7 |
| Half Astern: | 70 RPM/Pitch | 9.9.9 | 10.3 |
| Full Astern: | 80 RPM/Pitch | 10.5 75 % ahead power |  |
| Engine Critical RPM: 55 ~ 67 RPM | Maximum Number of Consecutive Engine Starts: 12 | Time from Full Ahead to Full Astern: 480 sec |  |
| Time Limit Astern: 30 min |  | Maximum Steering Speed: |  |

| STEERING |  |  |  |
| --- | --- | --- | --- |
| Number of Propellers: 1 | Direction of Turn: right-handed |  | Propeller Arrangement: |
| Time from Hard-Over to Hard-Over: 22 sec |  | Rudder Angle for Neutral Effects: 0 |  |
| Thrusters (Positions and Power): N/A |  | Steering Characteristics: |  |
| EQUIPMENT CHECKED AND READY FOR USE |  |  |  |
| Anchors: | ✓ |  | Cleared Away: YES/NO |
| Compasses: | ✔ |  |  |
| Compass Error: | ✔ |  |  |
| Speed Log: | ✔ |  | Doppler: YES/NO Speed: Water/Ground Axis: Single/Dual |
| Echo Sounder: | ✔ |  |  |
| GNSS: | ✔ |  | Type: DGPS |
| ECDIS: | ✔ |  |  |
| X-Band Radar: | ✔ |  | ARPA: YES/NO |
| S-Band Radar: | ✔ |  | ARPA: YES/NO |
| VHF (Including Handheld): | ✔ |  |  |
| Steering Gear: | ✔ |  | Number of Power Units In Use: 2 |
| Engine Telegraphs: | ✔ |  |  |
| Rudder/RPM/ROT Indicators: | ✔ |  |  |
| Mooring Winches and Lines: | ✔ |  |  |
| Navigation Lights: | ✔ |  |  |
| Whistles: | ✔ |  |  |
| VDR: | ✔ |  |  |
| EQUIPMENT OPERATIONAL DEFECTS |  |  |  |
| N/A |  |  |  |
| OTHER IMPORTANT DETAILS |  |  |  |
| N/A |  |  |  |

Master: ___CAPT. SOE PYNE KYAW____				Date: _       07-02-2026___