| VESSEL / PILOT INFORMATION EXCHANGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M/V |  |  |  |  | ST. AIDAN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PORT: |  |  |  |  | Marsden Point |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DATE: |  |  |  | 08.10.2024 |  |  |  |
| A. DIMENSIONS / TONNAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gross: | 22 667MT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Net: |  |  | 12 351MT |  |  |  |  |  |  |  |  |  |  |  |  | Summer Draft: |  |  |  |  |  |  |  | 10.028 |  |  |  |  |  |
| Present Displacement : |  |  |  |  |  |  |  |  |  |  |  |  | 38853mt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Present DWT : |  |  |  |  |  |  |  |  | 30142 mt |  |  |  |  |  |  |  |  |  |
| L.O.A.: | 182.87 M |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | L.B.P. : |  |  |  |  | 179.95 M |  |  |  |  |  |  |  |  |  |  | BEAM: |  |  |  | 31.00 M |  |  |  |  |  |  |  |  |  |
| Bridge to Bow : |  |  |  |  |  |  |  | 157.15 M |  |  |  |  |  |  |  |  | Birdge to Manifolds : |  |  |  |  |  |  |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |  | Bridge to Stern : |  |  |  |  |  |  |  |  | 25.72 M |  |  |  |  |
| Draft FWD : |  |  |  |  |  |  | 8.37M |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Draft MID : |  |  |  |  |  |  |  |  | 8.46 M |  |  |  |  |  |  | Draft AFT: |  |  |  |  |  | 8.59 M |  |  |  |  |  |  |  |
| Calculated Squat at Full SEA speed: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.50 |  |  |  |  |  | Calculated Squat at manoeuvring Slow speed: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.81 |  |  |
| B. PROPULSION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type of Main Engine : |  |  |  |  |  |  |  |  |  |  |  |  | MAKITA-MITSUI-MAN B&W 6S46ME-B8.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | B.H.P. : |  |  |  | 8833.5 H.P. |  |  |  |  |  |  |  |  |  | Fuel : |  |  | VLSFO |  |  |  |  |
| Sea Condition RPM & Speed : |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 115 RPM/ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Critical RPM : |  |  |  |  |  |  | 59.0-71.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Notice Required for Manoeuvering : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 MIN |  |  |  |  |
| Bridge control: |  |  |  |  |  |  |  |  |  |  |  | YES / NO |  |  |  |  |  |  |  |  |  |  |  | Main Engine Telegraph presently at : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Restrictions on Engine Movements: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MANOEUVERING RPM / SPEED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FULL |  |  |  |  |  |  |  |  |  |  |  |  |  | HALF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SLOW |  |  |  |  |  |  |  |  |  |  |  |  | DEAD SLOW |  |  |  |  |  |  |  |  |  |
| RPM |  |  | SPEED |  |  |  |  |  |  |  |  |  |  | RPM |  |  |  |  |  | SPEED |  |  |  |  |  |  |  |  |  |  | RPM |  |  |  |  |  | SPEED |  |  |  |  |  |  | RPM |  |  |  |  |  |  | SPEED |  |  |
| 89 |  |  | 11.2 |  |  |  |  |  |  |  |  |  |  | 75 |  |  |  |  |  | 9.5 |  |  |  |  |  |  |  |  |  |  | 53 |  |  |  |  |  | 6.8 |  |  |  |  |  |  | 39 |  |  |  |  |  |  | 5.0 |  |  |
| C. MANOEUVERING PARTICULARS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BOW Thruster : YES / NO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Propeller Rotation : RIGHT / LEFT / C.P. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stopping Distance : |  |  |  |  |  |  |  |  |  |  |  | Full Speed: |  |  |  |  |  | 19,2 cables |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Slow Speed: |  |  |  |  | 4,6 cables |  |  |  |  |  |  |  |  |  |  |
| TURNING CIRCLE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full Speed : |  |  |  |  |  |  | 2,9 cables |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PORT |  |  |  |  |  |  |  |  | 2,9 cables |  |  |  |  |  |  |  |  |  |  |  | STBD |
| Slow Speed : |  |  |  |  |  |  | 4,1 cables |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PORT |  |  |  |  |  |  |  |  | 4,1 cables |  |  |  |  |  |  |  |  |  |  |  | STBD |
| For more details on ship's manoeuvring, if required, please see ZIG-ZAG and Manoeuvring characteristics tables posted in the Wheelhouse. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D. NAVIGATIONAL EQUIPMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gyro Compass Error: |  |  |  |  |  |  |  |  |  |  |  | 0.2E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | VHF on Channel(s) : |  |  |  |  |  |  |  |  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |
| RADAR : |  |  |  | X-BAND |  |  |  |  |  |  | cm |  |  |  |  |  |  |  | 3 CM |  |  |  |  | Display mode |  |  |  |  |  |  |  |  |  |  |  |  |  | Range: |  |  | 3 NM |  |  |  |  |  |  |  |  |  |  |  |  |
| RADAR : |  |  |  | S-BAND |  |  |  |  |  |  | cm |  |  |  |  |  |  |  | 10 CM |  |  |  |  | Display mode |  |  |  |  |  |  |  |  |  |  |  |  |  | Range: |  |  | 6 NM |  |  |  |  |  |  |  |  |  |  |  |  |
| ARPA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Equipment Deficiencies : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NIL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Speed Log Type: |  |  |  |  |  |  |  |  |  |  |  | DOPPLER SPEED LOG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| E. OTHER INFORMATION REQUIRED BY THE PILOT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notice Required for Crew to Standby FWD and AFT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 Minutes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F. INFORMATION TO BE OBTAINED FROM THE PILOT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number and position of mooring lines & Sequence of line Running / Letting go |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mooring / Unmooring and Towage Plan 31M M m 99.4 87.4 157.45m 25.77 m 46.38 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Air Draught 37.95 m Free Board 6.04 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UNDERKEEL CLEARANCE OPEN WATERS ((V2xCΒ)/100 in mtrs) Speed: ____13.5____________ knots SQUAT: ______1.50________ m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CONFINED WATERS (2x(V2xCΒ)/100 in mtrs) Speed: __________7.0_______ knots SQUAT: __________ 0.81_______m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of Tugs to be used : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Power of Tugs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tugs' or Ship's Lines |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Position and Time of Tugs Alongside: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Disposition of Tugs alongside : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Abort positions/ contingency anchorages : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lat: , Long: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Change of Pilot |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Passage Plan Discussed with Pilot: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Position of Disembarkation of Pilot : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No Deficiencies, No Defects |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Master’s Name: |  |  |  |  |  |  |  |  |  | SVERBA YEVHEN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Pilot's Name: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Master’s Signature: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Pilot's Signature: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |