Pilot	Card

| Voy No.: 2601 | Port & Country : MARSDEN POINT, NZ |  |  | Date: 2026.01.13 |  |
| --- | --- | --- | --- | --- | --- |
| Agent Details: Inchcape |  |  |  |  |  |
| SHIP PARTICULARS |  |  |  |  |  |
| Name: YANGTZE HAPPINESS |  |  |  | Call Sign: VRLA7 |  |
| Displacement: 19227 | DWT: 32377 |  |  | Year Built: 2012-08-02 |  |
| Length: 179.9M | Beam: 28.4M |  |  | Bulbous Bow: YES |  |
| Draught Fwd: 4.28 m | Draught Amidships: 5.28 m |  |  | Draught Aft: 6.28 m |  |
| Air Draught: 35.9 m | Port Anchor: 11 Shackles |  |  | Starboard Anchor: 11 Shackles |  |
| Bow Blind distance: 295.3M(IN BALLAST)/172.74M(LOADED) 1 shackle = 27.5 m/15 fathoms |  |  |  |  |  |
| 28.9m | 151.0m MManifold Parallel W/L Loaded 174.8m Ballast 118.30m |  |  |  |  |
| 28.4m |  |  | Air draught M |  | 42.1 m |
| MAIN ENGINE |  |  |  |  |  |
| Type: STX - B&W 6S42MC | Max Power: 6480 |  | kW | Max Power: 8686.3 | HP |
|  | RPM/Pitch |  | Loaded Speed (kts) | Ballast Speed(kts) |  |
| Sea Full Ahead(Limited by sea depth and EPL) | 117 |  | 12.1 | 12.5 |  |
| Harbor Full Ahead | 108 |  | 11.0 | 11.5 |  |
| Half Ahead: | 90 |  | 10.2 | 10.5 |  |
| Slow Ahead: | 75 |  | 7.5 | 8.5 |  |
| Dead Slow Ahead: | 50 |  | 5.0 | 5.5 |  |
| Dead Slow Astern: | 48 |  |  |  |  |
| Slow Astern: | 75 |  |  |  |  |
| Half Astern: | 83 |  |  |  |  |
| Full Astern: | 90 |  | 50% ahead power |  |  |
| Engine Critical RPM: 56-69 | Maximum Number of Consecutive Engine Starts: 12 |  |  | Time from Full Ahead to Full Astern: 420 s |  |
| Time Limit Astern: 2.4 MIN |  | Minimum Steering Speed: 4.5kt |  |  |  |

Cont.-

| STEERING |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Propellers: 1 |  | Direction of Turn: Right Handed |  |  |  |  | Propeller Arrangement: CENTRE |  |  |  |  |
| Time from Hard-Over to Hard-Over: 11S/23S |  |  |  | Rudder Angle for Neutral Effects: 0 degrees |  |  |  |  |  |  |  |
| Thrusters (Positions and Power): N/A |  |  |  | Steering Characteristics: SEMI-SPADE |  |  |  |  |  |  |  |
| EQUIPMENT CHECKED AND READY FOR USE |  |  |  |  |  |  |  |  |  |  |  |
| Anchors: | YES |  |  |  |  |  |  | Cleared Away: YES/NO |  |  |  |
| Compasses: | YES |  |  |  |  |  |  |  |  |  |  |
| Compass Error: | 0 |  |  |  |  |  |  |  |  |  |  |
| Speed Log: | YES |  |  |  |  |  |  | Doppler: YES/NO Speed: Water/Ground Axis: Single/Dual |  |  |  |
| Echo Sounder: | YES |  |  |  |  |  |  |  |  |  |  |
| GNSS: | YES |  |  |  |  |  |  | Type:GPS |  |  |  |
| ECDIS: | YES |  |  |  |  |  |  | FURUNO |  |  |  |
| X-Band Radar: | YES |  |  |  |  |  |  | ARPA: YES |  |  |  |
| S-Band Radar: | YES |  |  |  |  |  |  | ARPA: YES |  |  |  |
| VHF (Including Handheld): | YES |  |  |  |  |  |  |  |  |  |  |
| Steering Gear: | YES |  |  |  |  |  |  | Number of Power Units In Use: 2 |  |  |  |
| Engine Telegraphs: | YES |  |  |  |  |  |  |  |  |  |  |
| Rudder/RPM/ROT Indicators: | YES |  |  |  |  |  |  |  |  |  |  |
| Mooring Winches and Lines: | YES |  |  |  |  |  |  |  |  |  |  |
| Navigation Lights: | YES |  |  |  |  |  |  |  |  |  |  |
| Whistles: | YES |  |  |  |  |  |  |  |  |  |  |
| MOORING CONFIGURATION (Indicate On Diagram Below) | FWD |  | NBR |  |  |  |  | AFT |  | NBR |  |
|  |  |  | Head |  |  |  |  |  |  | Astern |  |
|  |  |  | Breast |  |  |  |  |  |  | Breast |  |
|  |  |  | Springs |  |  |  |  |  |  | Springs |  |
|  |  |  |  |  |  |  |  | (OBTAIN FROM PILOT) |  | TONNES | TUG NAME |
| MOORING DETALS/PLAN |  | MOORING WINCHES |  | 2+2 |  |  |  | TUG’S BOLLARD PULL | NO.1 |  |  |
|  |  | MOORING BOLLARD |  | SWL |  | 64T |  |  | NO.2 |  |  |
|  |  | CLOSED CHOCK |  | SWL |  | 48T |  |  | NO.3 |  |  |
|  |  | ROLLER FAIRLEAD |  | SWL |  | 48T |  |  | NO.4 |  |  |
| WHEN BOLLARD PULL OF THE TUGS EXCEED THE SWL OF SHIPS MOORING BOLLARDS PILOT/TUGS SHOULD BE ADVISED. |  |  |  |  |  |  |  |  |  |  |  |
| EQUIPMENT OPERATIONAL DEFECTS / OTHER IMPORTANT DETAILS |  |  |  |  |  |  |  |  |  |  |  |
| NIL |  |  |  |  |  |  |  |  |  |  |  |
| OTHER IMPORTANT DETAILS |  |  |  |  |  |  |  |  |  |  |  |
| RPM PERFORM NOT MORE THAN 119.2 (Corresponding to 4360 KW) AS PER REQUIREMENT OF THE EPL&OMM. ABOUT FIVE MINUTES TO DISABLE THE EPL. |  |  |  |  |  |  |  |  |  |  |  |
| Completed by: Verified by : Acknowledged by: Name : Master Pilot Rank : Name : XIONG WEIMENG Name： Date: |  |  |  |  |  |  |  |  |  |  |  |