SHIP’ NAME:  TS HONOUR                VOYAGE NO.: 2502

| Port Name: MARSDEN POINT, NEW ZEALAND |  |  |  |  |  | Date 29 MARCH 2025 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Call sign V7HP8 Draught aft 6.96 m/ |  |  |  | Dead weigh 14760 tones Forward 4.49 m |  |  | Year built 2017 Displacement 24012 tones |  |  |  |
|  | ## SHIP’S PARTICULARS Length overall 182.00 m, Anchor chain: Port 11 shackles, Starboard 12 shackles, Breadth 30.00 m Stern \ shackles Bulbous bow Yes/No (1 shackle = 27.5 m/ 15 fathoms) |  |  |  |  |  |  |  |  |  |
| 25.3m 156.7m 30m 44.94m 37.98M Loaded 98.6m Ballast 89.1m |  |  |  |  |  |  |  |  |  |  |
|  |  | Type of engine: WARTSILA 5RT-flex50-D Tier II Delta Tuning Max. power 7044 kW ( HP) |  |  |  |  |  |  |  |  |
|  |  | Manoeuvring engine order | Rpm |  | Speed (knots) |  |  |  |  |  |
|  |  |  |  |  | Loaded |  |  | Ballast |  |  |
|  |  | Full ahead | 72 |  | 10.0 |  |  | 10.5 |  |  |
|  |  | Half ahead | 50 |  | 7.0 |  |  | 7.5 |  |  |
|  |  | Slow ahead | 44 |  | 5.5 |  |  | 6.0 |  |  |
|  |  | Dead slow ahead | 35 |  | 4.5 |  |  | 5.0 |  |  |
|  |  | Dead slow astern | -35 |  | Time limit astern |  |  | 30 min |  |  |
|  |  | Slow astern | -44 |  | Full ahead to full astern |  |  | 607 s |  |  |
|  |  | Half astern | -50 |  | Max. no. of consec. starts |  |  | 25 |  |  |
|  |  | Full astern | -72 |  | Minimum RPM 35 |  |  | 4.5 knots |  |  |
|  |  |  |  |  | Astern power |  |  | 70 % ahead |  |  |

|  |  | ## STEERING PARTICULARS Type of rudder : SEMI-SPADE/NACA Maximum angle 2 x 35° Hard-over to hard-over 22″55 s Rudder angle for neutral effect ±1.5° Thruster: Bow kW ( HP) Stern kW ( HP) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CHECKED IF ABOARD AND READY |  |  |  |  |  |  |  |
|  | Anchors |  |  | Indicators: |  |  |  |
|  | Whistle |  |  | Rudder |  |  |  |
|  | Radar  3cm |  |  10cm | Rpm/pitch |  |  |  |
|  | ARPA |  |  | Rate of turn |  Nil |  |  |
|  | Speed log  |  | Doppler: Yes/No | Compass system |  |  |  |
|  | Water speed |  |  | Constant gyro error | + 0.1° 。 |  |  |
|  | Ground speed |  |  | VHF |  |  |  |
|  | Dual-axis Nil |  |  | Elec. pos. fix. System |  |  |  |
|  | Engine telegraphs 2 |  |  | T ype : GP-150 ECDIS |  |  |  |
|  | Steering gear |  |  |  |  |  |  |
|  | Number of power 3 units operating |  |  |  |  |  |  |
| OTHER INFORMATION: Yes; No. 1; Is the ship equipped with an engine or shaft power limiter? EPL or ShaPoli; V. If yes, is the limiter mechanical or software based? Limiter mechanical / Software based. • If the ship is equipped with a mechanical limiter, has it been removed or disabled prior to the pilot boarding the ship?; N/A. • If the ship is equipped with a software-based limiter, will the ship’s crew be able to override it immediately at the request of the pilot?; V. 2; Can the main engine/s attain the posted manoeuvring power (RPMs) without delay?; V. 3; Are the master and all bridge navigational watchkeepers familiar with the override feature for the EPL?; V. 4; Does the vessel have a risk assessment with appropriate control measures in place for immediate access to the EPL when full manoeuvrability is needed for safe passage?; V. ZHANG ZHAOBIN Signature of Master Signature of Boarding Pilot |