NAV 03 – PILOT CARD

| Vessel Particulars |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Vessel: | PROPEL FORTUNE |  |  | Call Sign: | 9V7580 |
| Displacement: | 41025 | DWT: | 31391 | Year Built: | 2012 |
| Length: | 189.99 M | Beam: | 32.26 M | Bulbous bow: | NO |
| Draught FWD: | 7.84 M | Draught AFT: | 8.35 M | Draught MID: | 8.13 M |
| Air Draught: | 39.496 M | Port Anchor Shackles*: | 12 | Stbd Anchor Shackles*: | 11 |
| Freeboard: | 9.65 M |  |  |  |  |

*1 shackle = 27.4m / 15 fathoms

32.26 m 24.95 m 165.04 m 47.85 m m

| Main Engine |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type: | MITSUI MAN B&W 6S50MC-C |  |  |  |  |  |  |  | Max Power: (KW) |  |  |  | 8450 KW |  |  | Max Power: (HP) |  |  |  |  | 11329.63 HP |  |
| Engine Power Limiting Device Fitted? |  |  |  |  |  | Yes |  |  | EPL Max Power (KW): |  |  |  | 6333.32 KW |  |  | EPL Max Power (HP) |  |  |  |  | 8491.69 BHP |  |
|  |  |  |  |  |  |  | RPM / Pitch: |  |  |  |  |  | Loaded Speed (kts) |  |  |  |  | Ballast Speed (kts) |  |  |  |  |
| Full Ahead: |  |  |  |  |  |  | 88 |  |  |  |  |  | 11.1 |  |  |  |  | 11.9 |  |  |  |  |
| Half Ahead: |  |  |  |  |  |  | 78 |  |  |  |  |  | 9.9 |  |  |  |  | 10.6 |  |  |  |  |
| Slow Ahead: |  |  |  |  |  |  | 58 |  |  |  |  |  | 7.4 |  |  |  |  | 7.9 |  |  |  |  |
| Dead Slow Ahead: |  |  |  |  |  |  | 45 |  |  |  |  |  | 5.6 |  |  |  |  | 6.1 |  |  |  |  |
| Dead Slow Astern: |  |  |  |  |  |  | 45 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Slow Astern: |  |  |  |  |  |  | 58 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Half Astern: |  |  |  |  |  |  | 78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full Astern: |  |  |  |  |  |  | 88 |  |  |  |  |  |  |  |  |  |  | 60 % Full ahead power |  |  |  |  |
| Engine Critical RPM: |  | 62-74 |  |  |  |  |  | Maximum number of consecutive engine starts: |  |  |  |  |  | 12 |  | Time from full ahead to full astern: |  |  |  |  |  | 350 sec |
| Time Limit Astern: |  |  |  | 30 |  |  |  |  |  |  |  |  | Maximum Steering Speed: |  |  |  |  |  |  |  |  |  |
| Fuel Types |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| At Sea |  |  |  |  |  |  |  |  | Non ECA: VLSFO |  |  |  |  |  |  | ECA: LSMGO |  |  |  |  |  |  |
| In Port |  |  |  |  |  |  |  |  | Non ECA: VLSFO |  |  |  |  |  |  | ECA: LSMGO |  |  |  |  |  |  |
| Steering |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of Propellers: |  |  | 1 |  |  |  |  |  |  | Direction of Turn: |  | RIGHT HAND |  |  | Propeller Arrangement: |  |  |  |  | CENTRE |  |  |
| Time from hard over to hard over: |  |  |  |  | 21 SEC |  |  |  |  |  | Rudder Angle for Neutral effects: |  |  |  |  |  | 0 |  |  |  |  |  |
| Thrusters (positions and power): |  |  |  |  | NA |  |  |  |  |  | Steering characteristics / rudder type: |  |  |  |  |  | SEMI BALANCED |  |  |  |  |  |

| Squat |  |  |  |
| --- | --- | --- | --- |
| UKC (m) | Speed (kts) | Open Water | Narrow Channel |
| 1.22 | 2 | 0.029 | 0.041 |
| 1.23 | 3 | 0.065 | 0.093 |
| 1.24 | 4 | 0.116 | 0.166 |
| 1.25 | 5 | 0.182 | 0.260 |
| 1.26 | 6 | 0.261 | 0.373 |
| 1.27 | 7 | 0.356 | 0.509 |
| 1.29 | 8 | 0.465 | 0.665 |
| 1.31 | 9 | 0.588 | 0.841 |
| 1.33 | 10 | 0.726 | 1.038 |
| 1.35 | 11 | 0.879 | 1.257 |
| 1.37 | 12 | 1.046 | 1.496 |
| 1.40 | 13 | 1.228 | 1.756 |
| 1.43 | 14 | 1.424 | 2.036 |

Equipment / Operational Defects

All equipment are in good working order

Other Important Details (e.g. ship windage area, position of the Automatic Identification System (AIS) antenna, safe working load (SWL) of bollards)

Location of AIS Antenna – Fwd (port) on the monkey Island SWL of Bollards: - Centre (fwd & aft) – 78t Forward shoulder and quarter – 61t

| Master Name: | CAPT. ARUN KUMAR MANAYATH | Master Signature:: |  |
| --- | --- | --- | --- |
| Date: | 28.09.2024 | Time: | 2000 LT |