	Wisdom Marine International Inc.
慧洋國際企業股份有限公司



Pilot Card
領港卡

	SHIP PARTICULAR

	Vessel Name: Daiwan Justice
	Port: Panama

	Gross tonnage: 21525
	Net tonnage:11201
	Call Sign: HOZK

	Displacement (K/Tons):  22201        
	DWT (K/Tons):  13871        
	Year built: 2015.Aug.05

	Length (L.O.A.): 179.96M
	Beam: 30M
	Bulbous bow: Yes

	Draught Fwd:   6.72       
	Draught Aft:   8.91       
	Draught amidships:   7.86       

	Air draught:   33.04       
	Port anchor(shackles):11
	Stbd anchor(shackles):11

	(1 Shackle = 27.5m/15 fathoms)

	
[image: image1.wmf]Air

Draft

___m

___m

____ft ____in

___m

___m

___m

Parallel

W/L

Loaded _______m

Ballast _______m



	MAIN ENGINE

	Type:

Man B&M 6S46me-B8.3

	Main Engine Power Status:
 FORMCHECKBOX 

Max Power with EPL:
 4900

(KW);
(HP)
 FORMCHECKBOX 

Max Power without EPL:
 6260

(KW);
(HP)
In case of emergency, EPL may be overridden and Master shall be informed
How much time is required to remove EPL? 
5
(mins)
  

	
	RPM/Pitch
	Loaded speed (kts)
	Ballast speed (kts)

	Full Ahead:
	75
	10.9
	11.7

	Half Ahead:
	55
	8.4
	9.1

	Slow Ahead:
	45
	7.0
	7.7

	Dead Slow Ahead:
	36
	5.7
	6.4

	Dead Slow Astern:
	36
	

	Slow Astern:
	45
	

	Half Astern:
	55
	

	Full Astern:
	75
	Astern about 27% Full ahead power

	Engine Critical RPM:
        61-73RPM
	Max. No. of Consecutive Starts (Times):
12
	Time from full ahead to full astern (Min.):329S

	Time limit astern(Min.):
Limitless
	Minimum steering speed:
RPM32.5/5.2kt

	STEERING PARTICULARS

	Number of propellers:1
	Direction of turn: Starboard

	Max. angle: 35.8
	Propeller arrangement: 5-Blades,5600mm*4376.9mm

	Time form hard-over to hard-over (Sec.): 22.9S
	Rudder angle for neutral effect (Deg.):0

	Thruster (position and power):NIL
	Steering characteristics/ rudder type: 
SEMI-BALLANCED HANGING


	EQUIPMENT CHECKED AND READY FOR USE

	Anchors:
	Yes
	Cleared away: YES/NO

	Compass:
	Yes
	

	Compass error:
	+-0.3°
	

	Speed log:
	Yes/Water
	Doppler: YES/NO

Speed: Water/Ground

Axis: Single/Duel

	Echo sounder
	Yes
	

	GNSS:
	
	Type:

	ECDIS:
	Yes
	

	X-Band radar:
	Yes
	ARPA: YES/NO

	S-Band radar:
	Yes
	ARPA: YES/NO

	VHF (including handheld):
	Yes
	

	Steering gear:
	Yes
	Number of Power units in use:

	Engine telegraphs:
	Yes
	

	Rudder/rpm/rot. Indicators:
	Yes
	

	Mooring winches and lines:
	Yes
	

	Navigation lights:
	Yes
	

	Whistles:
	Yes
	

	EQUIPMENT OPERATIONAL DEFECTS (checked by WMC-09 / WMC-10)

	NIL

	OTHER IMPORTANT DETAILS 

(e.g. ship wind age are, position of the Automatic identification System (AIS) antenna safe working load (SWL) of bollards)

	FORE SWL:52/64T
AFT SWL:52/64T

	Master’s Signature:
	
	Date:
	


Reference: IMO Resolution A.601(15) Provision and Display of Maneuvering Information On Board Ships

116.45 m





40.9m





30.0m





152.72m





27.24m








	
	Version：1.5 / 2025.01.31
	1 / 2
	WMR-OT-42



_1479041546.doc


Ballast _______m







Loaded _______m







Parallel      W/L







___m







___m







___m







____ft ____in







___m







___m







Draft







Air












